Computer Architecture Lab

(EL-2012)

LABORATORY REPORT

Fall 2026

(Lab#03)

Apply Behavioral-level Verilog to Design and Simulate Combinational Modules

Student Name:   Hanzala Saeed    Roll No: 24i-6535   Section: CE-A

Date performed: 9/1/2026

MARKS AWARDED: ________ / 10

Signature and date of Instructor: _________________

Lab Instructors: Dr. Nasir Ali Shah, Engr. Fasih Ahmad

______________________________________________________________________________________________________________________________________________________

NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES, ISLAMABAD

| Prepared by: | Fasih Ahmad |  | Version: 1.0 |
| --- | --- | --- | --- |
| Verified by: | Dr. Nasir Ali Shah |  | Updated: Fall 2026 |

Task 1:

Description:

I made a 4-bit adder/subtractor with Verilog. A control signal tells the circuit whether to add or subtract. I used XOR gates to flip the operand only when subtraction is needed and I fed this flipped data as a carry-in for the subtraction. I wrote a testbench to run through input combinations and check both results. I ran the simulation in ModelSim. I also checked the design in a Logisim circuit.

Code:

//4 Bit adder and subtractor with control input module addSub(result,A,B,select); input [3:0] A; input [3:0] B; input select; output reg [3:0] result; always @* begin if(select==0) result=A+B; else result=A-B; end endmodule module testAdderSubtractor(); reg [3:0] A; reg [3:0] B; reg select; wire [3:0] result; addSub uut(result,A,B,select); initial begin A=4'b0011; B=4'b0001; select=0; #50 A=4'b0101; B=4'b0010; select=0; #50 A=4'b0110; B=4'b0011; select=1; #50 A=4'b1000; B=4'b0010; select=1; #50; end endmodule

Output:

Task 2:

Description:

I built a 2x1 multiplexer. The select line tells the device whether to output an AND or an OR of A and B. I wrote a testbench to run all combinations of A, B and Select and record the output. I simulated the design in ModelSim. I also ran a Logisim circuit to validate the result.

Code:

//2x1 MUX module mux2x1(y,a,b,select); input a; input b; input select; output reg y; always @* begin if(select==0) y=a&b; else y=a¦b; end endmodule module testMux2x1(); reg a; reg b; reg select; wire y; mux2x1 uut(y,a,b,select); initial begin a=0; b=0; select=0; #50 a=0; b=1; select=0; #50 a=1; b=0; select=0; #50 a=1; b=1; select=0; #50 a=0; b=0; select=1; #50 a=0; b=1; select=1; #50 a=1; b=0; select=1; #50 a=1; b=1; select=1; #50; end endmodule

Output:

Task 3:

Description:

I made a 2-bit comparator. It compares two 2-bit numbers and tells if they are equal if A is greater or if A is less than B. I wrote a testbench that covered every input pair to confirm the equal, greater and less outputs. I simulated the design in ModelSim. I built the same circuit, in Logisim and checked it.

Code:

module comparator2bit(A,B,equal,greater,less); input [1:0] A; input [1:0] B; output equal; output greater; output less; assign equal = (A==B); assign greater = (A>B); assign less = (A<B); endmodule module testComparator2bit(); reg [1:0] A; reg [1:0] B; wire equal; wire greater; wire less; comparator2bit uut(A,B,equal,greater,less); initial begin A=2'b00; B=2'b00; #50 A=2'b00; B=2'b01; #50 A=2'b01; B=2'b00; #50 A=2'b01; B=2'b01; #50 A=2'b10; B=2'b01; #50 A=2'b01; B=2'b10; #50 A=2'b10; B=2'b10; #50 A=2'b11; B=2'b10; #50 A=2'b10; B=2'b11; #50 A=2'b11; B=2'b11; #50; end endmodule

Output: